Cisco Switch setup

Setting up the switch we first started with a basic setup which included, renaming the switch to S1, logging trap debugging to 192.168.100.100, we set the line console and vty with password at logins, NTP server with an IP of 192.168.100.100. From here we have added two Vlan’s 50 and 100. They have been assigned accordingly FE0/1 has trunk enabled with both Vlan’s, FE0/2 and 0/4 have been assigned on Vlan 100 while 0/3 is Vlan 50 which our DMZ has been assigned on.

Palo Alto setup

For our Palo Alto setup we had a pre-defined setup to build upon it included 3 interfaces and zones, Our WAN is on our Ethernet1/1, LAN on E1/2 which is running a DHCP server, and E1/3 is our DMZ. The interfaces have been setup with an interface type as layer 3. Zones are set as inside (LAN), outside (internet) and DMZ, a default virtual router has been setup to connect everything together.

Two NAT policies have been created one called masquerade for WAN and DMZ to WAN, a dynamic-ip-and-port and been chosen for these policies and connected to ethernet1/1. An interface management has been setup so we are able to connect via HTTPS and SSH, network services include pink and response pages.

Policy security has been set in place so we have some piece of mind in making everything is as secure as possible, some of these polices include: LAN to DMZ, DMZ to WAN, Lan to WAN, DMZ to SIEM, and outside to DMZ remote MGT.

Security profiles in place include: Antivirus, Anti-spyware, Vulnerability Protection, URL Filtering, File Blocking, and Wildfire Analysis, hopefully with this all setup our network will be as secure as possible.

(to open and view XML log in git \opt\pancfg\mgmt\saved-configs\merged-running-config.xml)